Sensor Grid Project - Pico vs. ESP32

USDA ENG Sensor Group - Steven Pilgrim

Raspberry Pi Pico Power Efficiency:

The processor of the Pico has a relatively low clock speed and peak power consumption in active mode, resulting in power efficiency at the cost of processing speed. However, functioning in active mode the Pico would be significantly more power efficient relative to the ESP32 in its higher active modes. Below is a table based on the Pico datasheet explaining its power consumption.

| Power Mode | Descriptions | Power Consumption |
| --- | --- | --- |
| Active | Processor and peripherals running normally. Dynamic clock speed and power draw. | 90 ~ 95 mA. 87 ~ 92 mA. |
| Sleep | SRAM and peripherals remain active | 1.3 ~ 1.9 mA |
| Dormant | RTC and certain GPIO pins remain active | 0.8 ~ 1.4 mA |

ESP32 Power Efficiency:

The processor of the ESP32 has a relatively high clock speed and peak power consumption in active mode, resulting in faster processing at the cost of power inefficiency. However, functioning in deep-sleep mode the ESP32 would be significantly more power efficient relative to the Pico in any of its modes. Below is a table based on the ESP32 datasheet explaining its power consumption.

| Power Mode | Description | Power Consumption |
| --- | --- | --- |
| Active (RF Working) | Wi-Fi Tx packet 13 dBm ~ 21 dBm. Wi-Fi / BT Tx packet 0 dBm. Wi-Fi / BT Rx and listening. Association sleep pattern (by Light-sleep). | 160 ~ 260 mA. 120 mA. 80 ~ 90 mA. 0.9 mA (DTIM3), 1.2 mA (DTIM1). |
| Modem-sleep | The CPU is powered on | Max speed: 20 mA. Normal speed: 5 ~ 10 mA. Slow speed: 3 mA. |
| Light-sleep | - | 0.8 mA |
| Deep-sleep | The ULP co-processor is powered on. ULP sensor-monitored pattern. RTC timer + RTC memory. | 0.15 mA. 25 µA (1% duty). 10 µA. |
| Hibernation | RTC timer only | 2.5 µA |

Comparison:

In the context of this project, the micro-controller used for each sub-group will only stop being idle when it collects and sends data. Thus, the ESP32 is ideal for our project. However, should we need a micro-controller that is ideal for power efficiency when active, then the Pico is ideal.